# Auto Fill SABR Worksheet Version II

Wednesday, July 19, 2021

# Save the DVH Data

Open the DVH view in External Beam Planning

In the "Dose Statistics" Tab Right -Click on the "Show DVH" column

Select "Show DVH for all Structures"

DVH values will be calculated

Right-Click within the DVH

Select "Export DVH in Tabular Format…"

A “Save As” dialogue box will appear.

Select the folder:

M:\Dosimetry Planning Documents\SABR Plan Evaluation\DVH Files

Change "Save as type" to "All Files"

Enter a file name ending with ".dvh"

# Run the Plan Evaluation program,

Run "PlanEvaluation"

The main window appears:

Select the dvh file from the list in the bottom left of the window.

- (This is a list of all dvh files in the directory:

- M:\Dosimetry Planning Documents\SABR Plan Evaluation\DVH Files directory)

Click on "Load Plan" to import the dvh file data

The dvh file will load and the button label will change to “Plan Loaded”.

|  | - → |  |
| --- | --- | --- |

Select the appropriate Report from the drop-down menu in the top right.

Click on the “Match Structures” button.

A new window will pop up allowing you to Match the Plan Report Items with structure names.

The items are grouped by whether the auto-match was successful or not.

To manually match a structure to a given report item:

Right click on the Click on the desired item and hold the mouse over the Match heading on the menu that appears.

Click on the appropriate structure from the list:

It is also possible to manually type in text:

- Match Structures” button.

Selecting “Enter Value” from the right-click menu brings up a new window for typing in the text value

The value entered will display in the Matched Item column:

To remove a match, select “Clear” from the right-click menu”

The value in the Matched Item column will become “None”:

Once all desired modifications are complete, click on the Approve button in the lower left corner.

Click on “Generate Report”

The SABR Plan Report Form will appear and begin to populate

# Review Results

There is a slight difference from that standard form in that values for both Trachea and Bronchus appear, but the maximum of the two goes in the usual spot.

Clicking on any of the green buttons, selecting a new dvh file or plan report will move back to that step in the process.

# Note

This program is a work in progress.

Some things will not work as intended

Your feedback / bug reports / feature requests are very welcome